Manual

##### Sectn48

Version 1.1

C. Lugtmeier © 1999

## Contents

1.	Copyright and disclaimer of warranty	3

2.	Description	3

3.	Coordinate system and signs	3

4.	Overview	4

5.	Command reference	4

6.	Theory	8

7.	Examples	8

A.	Installation	12

B.	Characteristics	12

C.	History	12

D.	Author	12

## Copyright and disclaimer of warranty

All the files of the Sectn48 library are copyrighted © by C. Lugtmeier. Sectn48 is distributed in the hope that it will be useful, but the copyright holder provides the program “as is” without warranty of any kind, either expressed or implied, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. In no event will the copyright holder be liable to you for damages, including any general, special, incidental or consequential damages arising out of the use or inability to use the program.

This version of Sectn48 is a Giftware release. You may use it as long as you like, but only for non-commercial purposes and only as a private person. Permission to copy the whole, unmodified, Sectn48 library is granted provided that the copies are not made or distributed for resale (excepting nominal copying fees) and provided that you include on each copy this copyright notice and disclaimer of warranty.

## Description

The Sectn48 library is an application for the Hewlett Packard 48 series calculators. It is designed to calculate the properties of cross-sections.

Sectn48 can calculate almost all the characteristic properties of such a cross-section like the area, the center of area, the moment of inertia etc. Sectn48 can do this also for non-solid and oddly shaped cross-sections.

What’s more, Sectn48 can even calculate properties of a fictional (i.e. elastic) cross-section, that is, a cross-section composed of two materials. An example of this is a cross-section of a reinforced concrete beam. This was one of the main reasons for developing the Sectn48 library.

Sectn48 provides all of this through an easy to use interface. Additional features include a built-in plotter to check the correctness of the geometry of the cross-section. Sectn48 is completely written in SysRPL.

## Coordinate system and signs

Sectn48 uses an X-Y coordinate system where the positive X-direction is to the right and the positive Y-direction is upwards. Angles are positive counter-clockwise. Below the positive directions are drawn.

Note:

Since units are not supported with arrays on the HP48 you have to be careful about the values you enter. The implied units must be compatible for meaningful results. For instance if you enter X and Y coordinates in meters you have to enter the area in square meters.

## Overview

The Sectn48 library menu consists of six menulabels; SSECT, ESECT, SPLOT, EPLOT, CFGR and ABOUTSectn48. These commands will be explained shortly below.

- SSECT		Calculates properties of a cross-section

- ESECT		Calculates properties of an fictional (i.e. elastic) cross-section

- SPLOT		Plots the cross-section

- EPLOT		Plots the fictional (i.e. elastic) cross-section

- CFGR		Gives the Sectn48 configuration options

- ABOUTSectn48	The about screen with the version number

The SECT and PLOT  commands take their arguments from the stack in the form of real arrays and real numbers.

The basic idea is to describe a cross-section with a 2-column matrix where the first column contains the X-coordinates and the second column contains the Y-coordinates. The coordinates should describe a closed polyline.

For the calculation of properties of fictional cross-sections additional information is required. This should be provided in the form of the ratio of some material property and an array with X and Y coordinates and the precise area of the second material at this point.

The necessary arguments for the various commands and the properties of the commands themselves are described in depth in the next section Command reference.

## Command reference

SSECT

#### Standard section command: The SSECT command calculates properties of a cross-section.

Level 1			Level 20	Level 19	…	Level 1

[[matrix]]s		[[matrix]]s	A		…	’

Affected by Flags:

Userflag 40	clear: results to stack

set: S(X): display results per keystroke and results to stack, G(X): enter browser,

only marked results to stack

Remarks:

The [[matrix]]s is a {n 2} real matrix with column 1 as x-coordinates and column 2 as y-coordinates where the coordinates should describe a closed polyline (first row of [[matrix]]s equals last row of [[matrix]]s). The [[matrix]]s should have at least 3 rows (n  3).

Positive area is described counter-clockwise, negative area is described clockwise. This can be used to calculate the properties of non-solid cross-sections (see Examples).

The calculated properties are (from level 19 to level 1):

In the main x,y coordinate system ( reference point: (0,0) )

A	:	area

Xc	:	x-coordinate of “center of area”

Yc	:	y-coordinate of “center of area”

Sx	:	Static moment of area ( reference: X-axis )

Sy	:	Static moment of area ( reference: Y-axis )

Ix	:	Moment of inertia ( reference: X-axis )

Iy	:	Moment of inertia ( reference: Y-axis )

Ip	:	Polair moment of inertia ( reference point: (0,0) )

Ixy	:	Centrifugal moment of inertia ( reference: X and Y-axis )

I1	:	Main moment of inertia ( reference: X or Y-axis rotated over  )

I2	:	Main moment of inertia ( reference: X or Y-axis rotated over  )

	:	Angle of rotated (from X and Y axis)coordinate system for main moments

of inertia

In the translated x’,y’ coordinate system ( reference point: (Xc,Yc) )

Ix’	:	Moment of inertia ( reference: X’-axis )

Iy’	:	Moment of inertia ( reference: Y’-axis )

Ip’	:	Polair moment of inertia ( reference point: (Xc,Yc) )

Ix’y’	:	Centrifugal moment of inertia ( reference: X’ and Y’-axis )

I1’	:	Main moment of inertia ( reference: X’ or Y’-axis rotated over ’ )

I2’	:	Main moment of inertia ( reference: X’ or Y’-axis rotated over ’ )

’	:	Angle of rotated (from X’ and Y’ axis) coordinate system for main moments

of inertia

Note that the tag of I1 and I1’ (when necessary) will be enhanced by an indicator:

X (reference is X-axis), Y (reference is Y-axis), 1Q (reference is axis in first quadrant, rotated over  (or ’) from X-axis) or 2Q (reference is axis in second quadrant, rotated over  (or ’) from Y-axis).

ESECT

#### Imaginary section command: The ESECT command calculates properties of a fictional (i.e. elastic) cross-section. This cross-section consists of two (!) materials. An example would be a cross-section of a reinforced concrete beam. Note that by the nature of the calculation the area of the second material should be small compared to that of the main material (see Theory). It is also understood that the second material is enclosed by the main material (n-1 in formulas, see Theory). The ESECT command uses the SSECT command internally.

Level 3		Level 2		Level 1 	

n		[[matrix]]m2	[[matrix]]s	

Level 10	Level 9		Level 8		Level 7		…	Level 1

n		[[matrix]]m2	[[matrix]]s	Ai		…	Iy’i

Affected by Flags:

Userflag 40	clear: results to stack

set: S(X): display results per keystroke and results to stack, G(X): enter browser,

only marked results to stack

Remarks:

n is the ratio between the material properties of the second material (i.e. steel reinforcement bars) and the main material (i.e. concrete). For elastic cross-sections this material property would be the modulus of elasticity (i.e. ne = Esteel/Econc).

The [[matrix]]m2 is a {n 3} real matrix with column 1 as x-coordinates, column 2 as y-coordinates and column 3 the areas of the second material. The [[matrix]]m2 can have any number of rows (n  1).

The [[matrix]]s is a {n 2} real matrix with column 1 as x-coordinates and column 2 as y-coordinates where the coordinates should describe a closed polyline (first row of [[matrix]]s equals last row of [[matrix]]s). The [[matrix]]s should have at least 3 rows (n  3).

Positive area is described counter-clockwise, negative area is described clockwise. This can be used to calculate the properties of non-solid cross-sections (see Examples).

The calculated properties are (from level 7 to level 1):

In the main x,y coordinate system ( reference point: (0,0) )

Ai	:	fictional area

Xci	:	fictional x-coordinate of “center of area”

Yci	:	fictional y-coordinate of “center of area”

Ixi	:	fictional moment of inertia ( reference: X-axis )

Iyi	:	fictional moment of inertia ( reference: Y-axis )

In the translated x’,y’ coordinate system ( reference point: (Xci,Yci) )

Ix’i	:	fictional moment of inertia ( reference: X’-axis )

Iy’i	:	fictional moment of inertia ( reference: Y’-axis )

SPLOT

#### Standard section plot command: The SPLOT command plots the polyline of a cross-section.

Level 1			Level 1

[[matrix]]s		[[matrix]]s

Affected by Flags:

Userflag 41	clear: normal plot

set: plot is rotated 90 degrees clockwise

Userflag 42	clear: plot only to ABUFF (stack grob)

set: plot to ABUFF and PICT

Remarks:

After plotting the program waits for a keystroke to resume the normal stack display, the only exception is the OFF key (rightshift ON).

The [[matrix]]s is a {n 2} real matrix with column 1 as x-coordinates and column 2 as y-coordinates where the coordinates should describe a closed polyline (first row of [[matrix]]s equals last row of [[matrix]]s). The [[matrix]]s should have at least 3 rows (n  3).

Positive area is described counter-clockwise, negative area is described clockwise. This can be used to calculate the properties of non-solid cross-sections (see Examples).

EPLOT

#### Imaginary section plot command: The EPLOT command plots a fictional (i.e. elastic) cross-section. This cross-section consists of two (!) materials. An example would be a cross-section of a reinforced concrete beam. The second material is plotted as a cross.

Level 2		Level 1			Level 2		Level 1

[[matrix]]m2	[[matrix]]s		[[matrix]]m2	[[matrix]]s

Affected by Flags:

Userflag 41	clear: normal plot

set: plot is rotated 90 degrees clockwise

Userflag 42	clear: plot only to ABUFF (stack grob)

set: plot to ABUFF and PICT

Remarks:

After plotting the program waits for a keystroke to resume the normal stack display, the only exception is the OFF key (rightshift ON).

The [[matrix]]m2 is a {n 3} real matrix with column 1 as x-coordinates, column 2 as y-coordinates and column 3 the areas of the second material. The [[matrix]]m2 can have any number of rows (n  1).

The [[matrix]]s is a {n 2} real matrix with column 1 as x-coordinates and column 2 as y-coordinates where the coordinates should describe a closed polyline (first row of [[matrix]]s equals last row of [[matrix]]s). The [[matrix]]s should have at least 3 rows (n  3). Positive area is described counter-clockwise, negative area is described clockwise.

CFGR

#### Configuration command: The CFGR command provides a way to configure the Sectn48 library.

Level 1			Level 1



Affected by Flags:

No, however it affects flags (Userflag 40,41 and 42)

Remarks:

The CFGR command provides an easy menu-based interface to modify the behavior of the SSECT, ESECT, SPLOT and EPLOT commands.

ABOUTSectn48

#### About command: The ABOUTSectn48 command shows the about message of the Sectn48 library.

Level 1			Level 1



Affected by Flags:

No

Remarks:

The current version, year of production and the name of the author are shown.

## Theory

Underneath a couple of formulas are shown which are used to describe the properties of a fictional cross-section, used by the ESECT command (the ones used for the SSECT command can be found in most textbooks). Note that all the properties of the second material are marked by: m2.

Ai=A+(n−1)⋅Am2

Xci=Xc⋅A+(n−1)⋅Am2⋅Xm2Ai		Yci=Yc⋅A+(n−1)⋅Am2⋅Ym2Ai

Ixi=Ix+(n−1)⋅Am2⋅Ym22		Iyi=Iy+(n−1)⋅Am2⋅Xm22

Ix'i=Ixi−Ai⋅Yci2				Iy'i=Iyi−Ai⋅Xci2

## Examples

In the next section some examples are provided to familiarize yourself with the Sectn48 library.

#### Example 1

Below an example of an I-profile is shown.

To calculate the properties of this cross-section you have to enter the X and Y coordinates of the cross-section. Since the area is positive you have to work counter-clockwise. I have chosen the intersection of the X and Y axis on the bottom left of the cross-section. This is entirely free, however a reasonable choice lightens the work!

In this example I start from point 1 (and finish there also). This is also free (I could have started at point 8, but then would have had to finish there).

Having agreed on all this, you have to enter the following ({13 2} sized) array on the HP48:

[ [   0   0 ] [ 280   0 ] [ 280  40 ] [ 160  40 ] [ 160 260 ]

[ 280 260 ] [ 280 300 ] [   0 300 ] [   0 260 ] [ 120 260 ]

[ 120  40 ] [   0  40 ] [   0   0 ] ]

With this array on level 1 of the stack  you can execute the SSECT command and, depending on your version of the library and its current configuration, after a slight pause you will either enter a browser ( a screen-wide choose box), see the output displayed on line 1 to 7 or just see the tagged results appear on the stack.

If you want to check the geometry of the entered cross-section you can execute the SPLOT command.

#### Example 2

This example shows the necessary input for the calculation of the elastic properties of a post-stressed concrete T-beam. This T-beam is drawn below with the dimensions in millimeters.

To calculate the properties of the elastic cross-section we have to know the modulus of elasticity of the concrete and the steel. For this example we take Esteel = 2.0*105 [Mpa] and Econcrete = 33500 [Mpa]. Thus the value of ne is 2.0*105 / 33500 = 5.97.

If we enter everything in meters we have to know Ap in meters too. So, 3*1200 [mm2] equals 3*.0012 [m2].

The input for the ESECT command is:

Level 3:

5.97

Level 2:

[ [ .4 .08 .0012 ] [ .5 .08 .0012 ] [ .6 .08 .0012 ] ]

Level 1:

[ [ .25 0 ] [ .75 0 ] [ .75 .2 ] [ .6 .35 ] [ .6 1.05 ]

[ 1 1.05 ] [ 1 1.2 ] [ 0 1.2 ] [ 0 1.2 ] [ 0 1.05 ]

[ .4 1.05 ] [ .4 .35 ] [ .25 .2 ] [ .25 0 ] ]

The same input can be used for the EPLOT command (although there is no level 3 argument needed).

#### Example 3

This example shows the necessary input for the calculation of the elastic properties of a non-solid pre-stressed concrete beam. This pre-stressed concrete beam is drawn below with the dimensions in millimeters.

To calculate the properties of the elastic cross-section we have to know the modulus of elasticity of the concrete and the steel. For this example we take Esteel = 2.0*105 [Mpa] and Econcrete = 33500 [Mpa]. Thus the value of ne is 2.0*105 / 33500 = 5.97.

If we enter everything in meters we have to know Ap in meters too. So, let us say that Ap is 18900 [mm2] equals 0.0189 [m2].

The input for the ESECT command is:

Level 3:

5.97

Level 2:

[ [ .59 .27 .0189 ] ]

Level 1:

[ [ 0 0 ] [ 1.18 0 ] [ 1.18 2.45 ] [ 0 2.45 ] [ 0 0 ]

[ .26 .43 ] [ .26 1.87 ] [ .34 1.95 ] [ .74 1.95 ]

[ .82 1.87 ] [ .82 .43 ] [ .74 .35 ] [ .34 .35 ]

[ .26 .43 ] ]

For the level 1 argument you have to remember that positive area is described counter-clockwise, negative area clockwise. The same input can be used for the EPLOT command (although there is no level 3 argument needed).

A note about the examples:

They are fairly straightforward. However, you can use the Sectn48 library for any kind of section like triangles, arcs, circular rings etc.

## Installation

The Sectn48 library can be installed in any available port on the calculator. However, it works fastest from a non-covered port, that is port 0 or 1. To install the Sectn48 library on your calculator do the following:

- Transfer the library to your calculator and place the library on the stack

- Place the port number you wish to store the library in on the stack

- Press the STO button

- Warmstart the calculator (press ON and C simultaneously) or shut it off and then turn it on again

- Purge the variable which still contains the library

If any of this gives you problems you should read the HP manual!

## Characteristics

| Sectn48 | Description |
| --- | --- |
| Version | 1.0G/1.0S |
| Library number | 1600 (decimal) |
| Size (bytes) | 5594 (1.0G) 5642 (1.0S) |
| Checksum | #C178h (1.0G) #4AB1h (1.0S) |
| Systemflags used | none |
| Userflags used | 40, 41 and 42 |
| Language used | Sys-RPL |

## History

The Sectn48 library version 1.0 was written during the fall of 1998 and is based upon an idea I got from seeing the SPKT program by Stefan Mueller. As great as this program is it has got one or two drawbacks. As it is written in User-RPL it is slow. Secondly, I didn’t like the way the calculation results were presented and the way the cross-section was plotted (in PICT).

However, one of the main reasons of writing my own version is that I wanted to have a program which could not only calculate a lot of properties of a normal cross-section but also calculate some properties of “elastic” cross-sections such as reinforced or pre-stressed concrete.

## Author

C. Lugtmeier

Raoul Wallenbergstraat 27

2033 AL  Haarlem

The Netherlands

<c.lugtmeier@bne.ballast-nedam.nl>

Please note that you should use the appropriate version for your calculator, depending on your version,  S(X) or G(X). Failing to do so could result in a “Memory Lost” event.

Used for fictional cross-section calcultions.