EXAMPLES FOR PROGRAM GRIDLOD2

Example 1

A simple example is considered, for which the results can be readily verified by manual calculations. A rectangular grid with 16 nodes is shown in Fig.4a.  Four point loads of 100 kN are located as shown in Fig.4b in local ordinates u, v.

(a) Grid								(b) Point loads

Fig.4 - GRID AND LOADING FOR EXAMPLE 1.

The grid data is in File GRID2EX1.GRD and the node co-ordinates are in File GRID2EX1.NOD.

Position 1 is a trivial load case, (R at X=2, Y=1.5) such that loads 1 and 2 coincide with nodes 106 and 107.  Load 3 falls mid-way between nodes 110 and 114 and load 4 falls at the centre of the element.  If the program is made to output the load allocation to various nodes, the answers can be verified by inspection.

In Position 2 the Reference Point is at X=3, Y=2.  The load positions are shown in Fig.4a, and the results are not difficult to check manually.

Example 2

A skew grid is shown in Fig.5.  The node co-ordinates are in File GRID2EX2.NOD.  A vehicle shown in Fig.6b is placed on it in two positions, as shown in Figs.6a and 6c.  It is input as a Prescribed Vehicle, with the last 2 axles of a T44 truck deleted.  Compute the equivalent node loads.

Fig.5 - GRID FOR EXAMPLE 2

The grid lies on the negative side of the Z axis and the Y axis is vertical.

The grid data is prepared in File GRID2EX2.GRD.  The triangular load element (TRIA) must be selected for this grid.  The four corner nodes 1, 8, 68 and 61 define the grid area.

The vehicle data in local co-ordinates is entered according to the U-V axes in Fig.6b and the origin serves as the reference point, R.  The Y axis points upwards, hence the load multiplier is -1 as all node loads are required to be negative.

The first load position of the vehicle is shown in Fig.6a, with global co-ordinates of the reference point noted in the figure and U axis horizontal.  The second position is shown in Fig.6c.  The X axis must be rotated by 39.07o towards the Z axis, to match the vehicle U axis, hence w = 39.07 (positive).  Wheels 1 and 2 fall outside the grid, which is reported by the program.

Fig.6 - VEHICLE LOCATIONS FOR EXAMPLE 2.

Example 3

A truss bridge is subjected to the railway loading shown in Fig.7.  Compute the equivalent node loads on the truss for two positions of the train, one with Wagon 2 at the left end and one with the leading locomotive axle at the right end.

Fig.7 - TRUSS AND LOADING FOR EXAMPLE 3.

The node co-ordinates must be given for the top chords of two trusses symmetrically placed, as shown in Plan, to enable use of the Rectangular load element.  Node identification numbers are as shown above and the corresponding node data is in File GRID2EX3.NOD.

To produce the wheel load data using the Data Forms, first generate 300-A-12 loading for 28 wheels, then switch to Prescribed load type and overwrite the wheel loads and ordinates for the locomotive wheels No.17 to 28.

For the first load position let the Reference Point co-ordinates be XR = 0.55 and YR = 0.  For the second position XR = 32.5 - 40.15 -0.001 = 7.651 and YR = 0; the 1mm correction is to ensure that the loco wheel falls over the truss and not outside.  The grid data is in File GRID2EX3.GRD.

Many more load positions would need to be run for a proper design of the truss.  This example shows how the program can be used to apply equivalent node loads to the panel points of a truss.  If it is desired to apply the wheel loads as member loads on the top chords, then the output of global co-ordinates of the wheels would help in locating the wheels on the appropriate members.

************